# HAZOP Guidewords, Deviations and Potential Causes

For use in HAZOP studies in the chemical / petrochemical industry and the steel & metals industry

This reference expands the continuous-process “HAZOP Guidewords Draft” (guidewords for each line, plus overview guidewords). It is a prompt list, not a substitute for team judgement: the team should still ask “what else could cause this?” at every node.

How to read the table: the Deviation is the guideword applied to the Parameter. Causes without a tag apply to both industries. [C] = mainly chemical / petrochemical.  [S] = mainly steel & metals (iron-making, DRI, EAF / BOF, ladle metallurgy, casting, rolling, finishing, non-ferrous).

Structure: Part A – process parameters for each line / node;  Part B – steel & metals specific parameters;  Part C – auxiliary guidewords for each line;  Part D – overview guidewords for the whole study.

# 1. Standard HAZOP Guidewords (IEC 61882)

| Guideword | Meaning | Typical deviations (chemical / steel & metals) |
| --- | --- | --- |
| NO / NOT / NONE | Complete negation of the design intent | No flow, no agitation, no cooling water, no inerting |
| MORE (HIGH) | Quantitative increase | More flow, high pressure, high temperature, high level, more reaction |
| LESS (LOW) | Quantitative decrease | Less flow, low pressure, low temperature, low level, low concentration |
| AS WELL AS | Qualitative increase – something extra is present | Water in molten metal, air in fuel gas, extra phase, impurities |
| PART OF | Qualitative decrease – only part of the intent is achieved | One component missing, wrong ratio, incomplete reduction |
| REVERSE | Logical opposite of the intent | Reverse flow, backflow into utility headers, reverse rotation |
| OTHER THAN | Complete substitution – something entirely different happens | Wrong material, wrong destination, abnormal operation, maintenance mode |
| EARLY / LATE | Relative to clock time | Tapping late, purge ended early, sample result late |
| BEFORE / AFTER | Relative to order or sequence | Oxygen on before lance in position, charging before furnace is drained of water |
| FASTER / SLOWER | Rate of a step or change | Casting speed too fast, heat-up rate too fast for refractory |

# 2. Parameter – Deviation – Potential Causes

| Parameter | Deviation | Potential Causes |
| --- | --- | --- |
| Part A – Process Parameters for Each Line / Node (Chemical and Steel & Metals) Apply to every line, vessel or node in turn (continuous process). Causes from the original draft are retained and expanded; industry-specific causes are tagged. |  |  |
| Flow | No flow | - Empty source vessel, feed tank, bin or gas holder - Wrong routing / incorrect valve line-up - Blockage, gas lock or vapour lock - Slip plate / spectacle blind left in or incorrectly installed - Non-return valve fitted the wrong way round - Burst pipe or large leak upstream - Equipment failure – control valve fails closed, isolation valve, pump or compressor trip, vessel failure - Incorrect pressure differential (downstream pressure higher than upstream) - Isolation in error (wrong valve closed, LOTO on wrong equipment) - Power failure; instrument air failure (valves go to fail position) - Control system / DCS / PLC failure; spurious trip - Line frozen or solidified (loss of heat tracing or jacket heating) - [C] Polymerisation, crystallisation or hydrate formation in the line - [S] Molten metal or slag frozen in launder, runner, tap hole or nozzle (skull formation) - [S] Blocked tuyere, lance tip, burner or porous plug - [S] Bridging / hang-up of burden, DRI, pellets or scrap in bins, chutes or shaft furnace - [S] Conveyor, feeder or vibratory feeder stopped (belt tear, drive trip, chute blockage) |
| Flow | Reverse flow | - Defective or passing non-return valve - Siphon effect - Incorrect differential pressure - Two-way flow in common headers - Poor isolation / passing valves - Emergency venting or depressurisation - Incorrect operation (valve opened out of sequence) - Pump failure or pump running in reverse (wrong rotation after motor rewiring) - [C] Process fluid backflow into utility lines (nitrogen, steam, cooling water, potable water) when utility pressure is lower than process - [S] CO-rich process gas (BFG, COG, BOF/LD gas, DRI top gas) backflowing into nitrogen, air or steam headers - [S] Flashback in fuel-gas, oxygen or oxy-fuel lines (flame arrestor missing or failed) - [S] Loss of water seal / seal pot allowing gas back-flow into the plant or atmosphere - [S] Reverse flow of cooling water due to loss of pump with elevated header (draining of cooling circuits) |
| Flow | More flow | - High level in upstream vessel (higher static head) - Increased pumping capacity (parallel pumps started, larger impeller fitted) - Increased suction pressure / reduced delivery head - Greater fluid density - Exchanger tube leaks (high-pressure side into low-pressure side) - Restriction orifice plates deleted or scale dislodged - Cross-connection of systems - Control faults – control valve fails open, bypass left open, wrong set point - Two-phase flow - Power failure (valves failing open) - [C] Reagent dosing pump over-delivery (wrong stroke setting) - [S] Excessive oxygen flow to lance, burners or tuyeres (flow controller fault, wrong recipe) - [S] Over-feeding of DRI, scrap, fluxes or alloys (feeder speed, weigh-hopper failure) - [S] Excess fuel gas to reheat furnace, ladle pre-heater or stove burners |
| Flow | Less flow | - Low level in upstream vessel - Line restriction; valve partly closed - Filter or strainer blockage - Pipe or vessel leakage - Defective pumps (worn impeller, cavitation, air ingress, seal failure) - Fouling of vessels, valves, restrictors or orifice plates - Density or viscosity problems - Incorrect specification of process fluid - Two-phase flow - [C] Catalyst bed plugging or high pressure drop - [S] Scale, silt or biological fouling in cooling passages of panels, roof, tuyeres, staves, lances or copper moulds - [S] Partly blocked tap hole or nozzle (low tapping rate) - [S] Dust build-up in off-gas ducts, fans, baghouses or venturi scrubbers |
| Flow | Misdirected flow (Other than – wrong destination) | - Cross-connection between systems; wrong hose connected - Road / rail tanker discharged into wrong storage tank - Drain or vent valves left open - Utility station connected to process (nitrogen hose left connected) - [S] Molten metal or slag spilled outside ladle, pot or mould (ladle misaligned, crane positioning error, tilting over-travel) - [S] Slag carry-over into ladle or tundish - [S] Off-gas diverted to the wrong system (flare / stack / gas holder change-over error) |
| Flow | As well as (additional material or phase in the flow) | - Gas or air entrainment in liquid; liquid carry-over in gas - Water in hydrocarbon or solvent - [C] Unreacted material carried forward to next stage - [S] Water, ice or snow carried into the furnace with wet scrap, DRI or fluxes - [S] Entrained dust or fines in process gas (cyclone or scrubber failure) - [S] Oil, grease or paint on scrap generating fume and fire |
| Pressure | More pressure | - Surge problems (water hammer, fast-closing valves) - Leakage from interconnected high-pressure system - Gas breakthrough (inadequate venting / pressure relief) - Isolation procedures for relief valves defective (relief valve isolated) - Thermal overpressure / flashing / boiling / external fire - Decomposition / gas evolution - Runaway reaction - Positive displacement pumps or compressors against closed outlet - Failed-open pressure control valves (PCVs) - Incorrect operation - Blocked outlet, blocked vent or blocked flare / stack - Thermal expansion of trapped (blocked-in) liquid - [S] Steam generation from water contacting molten metal or slag (water-cooled element leak, wet charge) - [S] Furnace or converter off-gas pressure excursion (loss of draft fan, damper closed, slopping) - [S] Blast-furnace top-pressure surge from burden slip / hanging; hot-blast stove valve fault - [S] Gas holder over-fill or seal failure (BFG, COG, LD gas) - [S] Hydraulic system overpressure (relief valve set wrongly, accumulator) |
| Pressure | Less pressure (vacuum) | - Generation of vacuum condition (pumping out with vent closed, steam-out then cooling) - Condensation - Gas dissolving in liquid - Restricted pump / compressor suction line - Undetected leakage - Vessel drainage - Power failure - Incorrect operation - Vacuum breaker or vent blocked or frozen - [S] Loss of furnace pressure control – air drawn into CO-rich or H2-rich system forming a flammable mixture - [S] Vacuum degassing (RH / VD / VOD) – vacuum pump or steam ejector failure, vessel seal leak - [S] Low fuel-gas pressure at burners (flame-out followed by re-ignition of accumulated gas) - [S] Low cooling-water pressure to water-cooled elements (pump trip, header leak) |
| Pressure | Pressure fluctuation / surge | - Water hammer; rapid valve closure; pump start/stop - Compressor or fan surge - Slug flow / two-phase flow - Control loop instability - [S] Converter slopping or foaming slag eruption - [S] Explosive spalling of wet refractory |
| Temperature | More temperature | - Ambient conditions (hot weather, solar gain) - Fouled or failed exchanger tubes - Fire situation - Cooling water failure - Defective control (TIC fault, heater fails on) - Reaction control failures - Power failure (loss of cooling fans or pumps) - Heat tracing or jacket heating fails on - [C] Exothermic reaction / catalyst hot-spot; loss of quench - [S] Loss of cooling water to panels, roof, tuyeres, staves, lances or moulds – burn-through - [S] Refractory wear or loss – shell hot-spot, breakout - [S] Excessive metal superheat (power-on time too long, temperature measurement error) - [S] Hot DRI / HBI self-heating and re-oxidation in bins, silos or conveyors - [S] Reheat furnace over-firing; burner management failure - [S] Hot material on conveyors not rated for the temperature (belt fire) |
| Temperature | Less temperature | - Ambient conditions (cold weather, freezing) - Reducing pressure (auto-refrigeration of LPG, CO2, ammonia) - Fouled or failed exchanger tubes - Loss of heating (steam, hot oil, electric tracing) - Power failure - Brittle fracture of carbon steel below its minimum design metal temperature - [C] Crystallisation, hydrate formation or high viscosity - [S] Metal freeze in ladle, tundish, launder or nozzle (low tap temperature, delays) - [S] Insufficient pre-heat of ladles, tundishes, moulds or tools – moisture and thermal shock - [S] Low reducing-gas temperature – incomplete reduction / carbon deposition |
| Level | High level | - Inlet flow greater than outlet; outlet blocked or valve closed - Level controller or level transmitter failure - Wrong vessel being filled - Overfill during transfer or tanker offloading - [C] Foaming - [S] Over-filling of ladle at tap (molten metal / slag spill) - [S] High mould level in continuous caster (overflow) - [S] Foaming slag over the sill / door - [S] Bin, silo or hopper over-fill (DRI, pellets, coal, lime) – spillage, structural overload |
| Level | Low level / no level | - Loss of feed; drain left open; leak - Level indication fails high (false reading) - Pump cavitation or loss of suction - Gas blow-by to downstream low-pressure vessel - [S] Low tundish level – slag entrainment, vortexing - [S] Low water level in seal pots or water seals – gas escape - [S] Low level in boiler drum or evaporative cooling system – overheating - [S] Empty bin or hopper allowing gas to pass back through feed system (loss of material seal) |
| Composition / Contamination | Contamination / composition change (As well as) | - Leaking exchanger tubes or isolation valves - Incorrect operation of system - Interconnected systems - Effect of corrosion - Wrong additives - Ingress of air, water or lubricant - Shutdown and start-up conditions - Phase change - Sediments / scale - Biological activity - Reaction by-products / breakdown products - [C] Catalyst poisons in feed - [S] Wet or contaminated scrap – sealed containers, explosives, oil, paint, plastics, radioactive sources - [S] Moisture in ferro-alloys, fluxes, ladle additions or tundish powders - [S] Oxygen or air ingress into reducing gas or inerted systems - [S] Hydrogen, CO or H2S in process gas above design values |
| Composition / Contamination | Part of (wrong ratio / concentration) | - Feed ratio controller failure; one feed lost - Dosing pump stopped or air-locked - Analyser or sample error - [C] Wrong acid or caustic concentration - [S] Carbon, sulphur or metallisation of DRI outside specification - [S] Wrong fuel / air ratio – incomplete combustion, CO formation - [S] Pickling acid strength out of range |
| Composition / Contamination | Other than (wrong material) | - Wrong raw material delivered or unloaded - Mislabelled drums, IBCs or cylinders - Wrong gas connected (e.g. oxygen in place of nitrogen, nitrogen in place of breathing air) - Incorrect hose / coupling standardisation - [S] Wrong alloy addition or wrong grade of scrap charged |
| Phase | Unintended phase change | - Flashing, boiling or condensation - Freezing or solidification - Two-phase flow in single-phase design - [C] Vapour lock; liquid in compressor suction - [S] Molten metal–water contact – steam explosion - [S] Solidification of slag or metal in transfer equipment |
| Reaction | More reaction / runaway | - Loss of cooling or agitation - Excess catalyst or initiator; wrong reactant order - Higher feed concentration or temperature - Accumulation of unreacted material followed by sudden reaction - [C] Contamination promoting decomposition - [S] Excessive oxygen blowing – violent carbon boil / slopping - [S] Metal–water reaction generating hydrogen - [S] Aluminium, magnesium or other reactive metal powders in contact with water |
| Reaction | Less / no reaction | - Low temperature; catalyst deactivated - Wrong or missing reactant; inhibitor excess - [C] Accumulation of unreacted material (later runaway) - [S] Incomplete reduction in DRI shaft / kiln (low metallisation) - [S] Low reducing-gas quality or temperature |
| Reaction | Unwanted side reaction (Other than) | - [C] Decomposition, polymerisation, oxidation - [S] Carbon deposition (Boudouard reaction) in reducing-gas systems - [S] Carbonyl formation where CO contacts nickel or iron surfaces at low temperature - [S] Re-oxidation of DRI / HBI (heat and hydrogen release) - Toxic gas evolution (H2S, HCN, NOx, phosphine from wetted ferro-silicon) |
| Mixing / Agitation | No / less / more mixing | - Agitator, mixer or circulation pump trip - Wrong agitator speed - [C] Stratification of reactants - [S] Argon / nitrogen stirring – porous plug blocked or leaking - [S] Excessive stirring – metal splash, slag entrainment - [S] Electromagnetic stirrer failure |
| Time / Sequence | Early / late / before / after / step omitted | - Procedure step skipped or done out of order - Batch timer or sequence controller failure - Purge time too short before light-up (burner management) - Operator distraction or shift changeover - [C] Reagent added before reaction mass at temperature - [S] Tapping late (over-oxidation, high temperature) or early (low temperature) - [S] Oxygen on before lance in correct position; lance not withdrawn at end of blow - [S] Charging scrap into a furnace with water present or hot heel too large - [S] Ladle not dried out / pre-heated before use |
| Time / Sequence | Faster / slower | - Ramp rate too fast for equipment or refractory - [S] Casting speed too fast – breakout - [S] Heat-up / dry-out of new refractory too fast – spalling - [S] Crane travel or tilting too fast – metal splash |
| Viscosity / Density | More / less viscosity or density | - Temperature change - Wrong material or concentration - Solids content too high - [S] Slag basicity or chemistry wrong – viscous slag, poor fluidity |
| Part B – Steel & Metals Industry Specific Parameters Additional parameters for iron-making, DRI, steel-making (EAF / BOF / ladle metallurgy), casting, rolling, finishing and non-ferrous metals. Use alongside Part A. |  |  |
| Molten metal / slag containment | Loss of containment (breakout, spill, splash) | - Refractory wear, erosion or spalling; missed refractory inspection - Furnace shell or ladle shell burn-through - Tap-hole or slide-gate failure; nozzle failure - Ladle over-filling - Crane failure or dropped ladle; ladle trunnion failure - Ladle or pot not correctly seated on car or stand - Tilting drive over-travel; loss of hydraulic or electrical control - Breakout at caster (mould level, casting speed, lubrication, sticker) - Vehicle or ladle-car collision; derailment |
| Molten metal / slag containment | Molten metal / slag contact with water (As well as) | - Leaking water-cooled panel, roof, tuyere, stave, lance or copper mould - Wet scrap, DRI, fluxes or ladle additions; snow or ice - Wet or damp ladles, tundishes, moulds, tools or slag pots - Water on floor or in pits under tapping areas - Rain ingress through damaged roofs; flooding - Spray cooling water impinging on spilled metal - Granulation / slag-pit water management failure |
| Water-cooled elements | No / less cooling water | - Pump trip or power failure without emergency (gravity / diesel) supply - Valve closed in error; header isolation - Scale, silt or biological fouling - Air locking of circuits - Low header pressure or tank level - Cooling tower failure (fans, basin level) |
| Water-cooled elements | Water leak into furnace / vessel | - Thermal fatigue cracking of panels or lance - Arc or flame impingement damage - Mechanical damage from scrap charging - Erosion or corrosion (water chemistry) - Poor weld or repair quality - Over-pressure of closed circuit - Leak detection (flow in/out differential) failed or not fitted |
| Process gas (CO, H2 – BFG, COG, LD gas, DRI reducing / top gas) | Toxic release (Other than / As well as) | - Flange, valve or seal leak; water-seal blown - Gas back-flow into buildings via drains or cable ducts - Bleeder / relief opening near work areas - Isolation failure during maintenance (no double block and bleed, no blinding) - Gas detection inoperative or poorly located - Confined spaces near gas mains |
| Process gas (CO, H2 – BFG, COG, LD gas, DRI reducing / top gas) | Explosive mixture (air or oxygen in gas) | - Air ingress through leaks at negative pressure - Inadequate purging at start-up / shutdown - Gas holder or collector low pressure - Oxygen analyser failure on LD gas recovery - Burner flame-out with continued fuel supply |
| Oxygen (lances, burners, oxygen pipelines) | Oxygen fire / oxygen enrichment | - Oil, grease or debris contamination in oxygen service - High gas velocity / particle impingement in valves and bends - Incompatible materials of construction or gaskets - Fast-opening valves (adiabatic compression) - Leak into enclosed areas – enriched atmosphere, clothing fire - Lance blocked or burned back |
| Inert gas (N2, Ar) | Asphyxiation / loss of inerting | - Leak or purge into enclosed or confined area - Inert gas used as breathing air (wrong connection) - Pit, tunnel or bin atmosphere not tested before entry - Loss of inert blanketing on DRI bins, coal mills or dust filters - Nitrogen supply failure (plant trip) |
| Electrical energy (EAF / LF / submerged arc / electrolysis) | More / less / other than intended | - Electrode breakage or slipping - Transformer or reactor cooling failure; transformer fire - Over-current or short circuit; regulation fault - Arc flash on high-current bus bars - Loss of furnace power at critical stage (metal freeze) - [S] Electrolysis cells / potlines – DC magnetic fields, anode effect, cell tapping errors |
| Solid charge materials (scrap, DRI / HBI, pellets, coke, coal, fluxes, ferro-alloys) | Wrong / contaminated / excess charge | - Sealed containers, gas cylinders, explosives in scrap - Radioactive sources in scrap (portal monitor failure) - Moisture in charge - Oversize pieces – bridging, mechanical damage - Segregation – inconsistent chemistry - Fines – dust generation |
| Hot DRI / HBI handling | Self-heating / fire / gas generation | - Re-oxidation in air (insufficient inerting or cooling) - Water contact – heat and hydrogen generation - Dust and fines accumulation - Conveyor or transport vessel over-temperature - Long storage times; poor bin ventilation |
| Combustible dust (coal / PCI, coke, DRI fines, Al / Mg / Ti powders, carbon) | Dust cloud / ignition | - Dust accumulation on structures; poor housekeeping - Dust collector or baghouse fire / explosion - Hot surfaces, sparks, static, hot work - Loss of inerting in coal mill or PCI system - Explosion venting / suppression / isolation missing or ineffective |
| Hydraulic and lubrication oil systems (mills, casters, furnace tilt, cranes) | Leak / fire / loss of pressure | - Hose or fitting failure near hot metal or hot product - Use of mineral oil instead of fire-resistant fluid near hot areas - Accumulator failure – uncontrolled movement - Loss of pressure – uncontrolled tilt, roll gap or clamp release - Contaminated oil – valve sticking |
| Mechanical handling (cranes, ladle cars, transfer cars) | Failure / collision / dropped load | - Crane brake, rope or hook failure while carrying molten metal - Power failure with ladle suspended - Anti-collision or limit switch failure - Overload; incorrect lifting accessory - Poor visibility from steam or fume |
| Casting and rolling | Casting speed / mould level / product handling deviations | - Mould level control failure – overflow or breakout - Secondary cooling water failure - Cobbles in rolling mill; product jumping guides - Roll breakage; drive failure - Scale-breaker high-pressure water hazards |
| Refractory | Less / failed refractory | - Wear beyond limit; inspection missed - Wrong refractory specification - Inadequate dry-out (moisture – spalling, steam) - Thermal shock from rapid heating or cooling - Chemical attack by slag |
| Acids, pickling, coating and galvanising | Concentration / temperature / contamination deviations | - Acid concentration out of range – fume, over-pickling - Acid regeneration plant upset – HCl release - Fume extraction failure - Wet or contaminated steel entering molten zinc bath – eruption - Flux or zinc ash handling exposures |
| Part C – Auxiliary Guidewords for Each Line / Node From the original draft, applied to each line after the process parameters. Deviations are phrased as what can go wrong. |  |  |
| Relief | Inadequate / inappropriate relief | - Relief philosophy not defined for all scenarios (fire, blocked outlet, runaway, tube rupture) - Type of relief device and its reliability - Relief valve discharge location unsafe (to work areas, ignition sources) - Pollution implications of discharge - Relief isolated or blocked; incorrect set pressure - [S] Explosion doors / bleeders on furnaces and gas mains – location and maintenance |
| Process control / instrumentation | Control or safety instrumented function failure | - Control philosophy unclear - Location of instruments unsuitable - Response time; set points of alarms and trips - Time available for operator intervention - Alarm and trip testing not done - Fire protection of instruments and cables - Electronic trip / control amplifiers - Panel arrangement and location - Auto / manual facility and human error - Alarm flooding; bypassed trips - [S] Instruments in high temperature, dust and EMF environments (temperature probes, level radars, leak detection) |
| Sampling | Sampling error / exposure | - Sampling procedure inadequate - Time for analysis result too long - Calibration of automatic samplers / reliability - Accuracy of representative sample - Diagnosis of result - Exposure of sampler to toxic, hot or pressurised material - [S] Molten metal sampling and temperature measurement at the furnace door or ladle |
| Corrosion / erosion | More corrosion / erosion | - Cathodic protection arrangements inadequate - Internal / external corrosion protection - Engineering specifications - Zinc embrittlement - Stress corrosion cracking (chlorides) - Fluid velocities too high - Corrosion under insulation - [S] Abrasive slurries, dust-laden gas, acid dew-point corrosion in off-gas ducts |
| Service failure | Loss of utility / service | - Failure of dry instrument air - Failure of steam / nitrogen / cooling water - Failure of hydraulic power / electric power - Telecommunications - Heating and ventilating systems - Computers / control system / network - [S] Loss of oxygen, argon or natural gas supply from air-separation plant or pipeline |
| Maintenance | Unsafe maintenance conditions | - Isolation, drainage, purging - Cleaning, drying - Slip plates - Access - Rescue plan - Training - Pressure testing - Work permit system - Condition monitoring - [S] Hot work near gas mains or dust; work under suspended loads; work inside cooled-down furnaces |
| Spare equipment | Unavailability / wrong spares | - Installed / non-installed spare equipment - Availability of spares - Modified specifications - Storage of spares - Catalogue of spares |
| Electrical | Ignition source / electrical hazard | - Static discharge - Earthing arrangements - Insulated vessels / equipment - Low-conductance fluids - Splash filling of vessels - Insulated strainers and valve components - Dust generation and handling - Hoses - Area classification - [S] High-current bus bars, induced currents and magnetic fields |
| Human factors | Operator error / omission | - Unclear or absent procedures - Poor labelling and identification - Workload, fatigue, shift handover - Alarm overload; poor HMI design - Inadequate training or competence |
| Part D – Overview Guidewords (to be done when all lines have been studied) Whole-system questions from the original draft. The Deviation column states the concern; the Potential Causes column lists what to review. |  |  |
| Simplicity | Unnecessary complexity | - Can we omit plant or simplify control? - Too many interlocks or bypasses |
| Output – reliability and bottlenecks | Less output / unreliability | - Least reliable components - Most limiting components |
| Efficiency – losses | Loss of material, energy or conversion | - Where will we lose material, conversion, yield or energy? |
| Construction | Construction hazards and impacts | - Demolition - Impacts on existing site, existing operations and environment - Size of equipment - Special hazards – chemicals / dangerous goods, to workers, structural - Personal protective equipment |
| Commissioning | Commissioning not controlled | - Broadly how it will be done - Sequence - What difficulties and hazards can be foreseen - Review of procedures - Signage / tags / locks - Responsibilities - [S] Refractory dry-out and first heat |
| Start-up | Unsafe start-up | - First start-up methods, difficulties, hazards - Routine start-up methods, difficulties, hazards - Philosophy of control (auto, manual, local) - Too many interlocks? - Procedures - Purging and gas-freeing before light-up |
| Shutdown (isolation, purging) | Unsafe shutdown | - Planned shutdown - Emergency shutdown - Difficulties, hazards - [S] Holding molten metal during an extended stop; emptying furnaces and ladles |
| Breakdown (including service failure) | Breakdown / loss of services | - Power failure - Air, steam, water, vacuum, fuel - Vents - Computer - Other - Major causes of breakdown - Emergency procedures and communications - Which way should valves move (fail-safe positions)? |
| Materials | Wrong / inadequate materials of construction | - Vessels - Pipelines - Pumps - Valves - Gaskets / seals - Protective coatings - Others (refractory, copper, hoses) |
| Utilities and services | Inadequate utilities | - Lighting - Compressed air; instrument air - Nitrogen - Water (cooling, process, service, potable) - Steam - Power - Oxygen |
| Toxicity | Toxic exposure | - Material safety data sheets - Handling procedures - Precautions - Exposure monitoring - Emergency procedures - Ventilation - [S] CO, metal fume, silica, PAHs from coke and pitch |
| Fire and explosion | Fire / explosion | - Broad overview - Protection, prevention, control - Location of safety showers, deluges - Location of shut-off valves and controls - [S] Molten metal explosions, dust explosions, gas explosions, hydraulic oil fires |
| Environmental impact | Release to environment | - Vapour emissions - Dust emissions - Effluent - Noise - Ground seepage - Waste minimisation - [S] Slag, mill scale, sludge, EAF dust disposal |
| Natural hazards | External natural event | - Earthquake - Flooding - Thunderstorm (lightning protection) - High winds - Sandstorms; extreme heat |
| Quality and consistency | Off-spec product / variability | - What will most cause quality to fail or vary? - How to prevent it |
| Procedures – development and documentation | Missing / inadequate procedures | - Operations - Maintenance - Confined space - Inspection and testing - Emergency - Engineering drawings |
| Safety equipment | Missing / inadequate safety equipment | - Personal protective equipment (PPE) - Fire detection - Fire fighting - Means of escape - Gas monitors - Breathing apparatus - Safety shower - Barriers / guards - [S] Heat-protective clothing; molten-metal splash protection |
| Egress / access | Inadequate egress or access | - Number of exit points - Emergency egress - Size – width, height, length - Obstructions - Lighting - Fire protection |
| Entry / exit points | Unsafe entry / exit arrangement | - Location - Arrangement - Communications - External impacts |
| Layout and facility siting | Domino / escalation effects | - Spacing between hazardous units - Occupied buildings near hazards - Vehicle routes near molten metal or gas mains - Simultaneous operations (SIMOPS) |